# Time Synchronisation protocol

How a device can synchronise is time

This document is for know how a device can be synchronise to a master clock

5/4/2016

Contents

I.	Method of synchronisation	2

II.	Protocol payload	2

III.	Type	2

A.	SYNC type=1	2

B.	Delay Request type=2	3

C.	Delay Response type=3	3

D.	ACK type=4	3

E.	Ping type=5	3

IV.	CRC	3

# Method of synchronisation

There is 2 time we unknow the offset and the delay of transmission.

For the offset we send a message with the timestamp (T1) of the master when the slaves receive the message he save his timestamp (T2).

Offset = T2-T1.

For the delay of transmission time the slave save the time he send a request( T3) when master receive the request he save the current timestamp(T4). He send this timestamp to the slave.

The delay of transmission= (T4-T3)

Header

Length

type

Data

CRC

ID

Protocol payload

- Header a fix Header for the device now that is this protocol (8 bit)

- Lenght length in bytes(8bit)

- ID (8 bit) it’s the unique id of the device.

- Type type of command see section II

- Data data depend of type command.

- CRC  :checksum (8bit)

# Type

There is a few types for the time synchronization

Type = 1

Data = currentTime

SYNC type=1

In sync type master send a broadcast.

Data send his timestamp.

Type = 2

Delay Request type=2

In delay request no have data.

The master send a delay response after

## Delay Response type=3

Type = 3

id(8)

TimeStamp (8)

The master request send the Request timestamp. This timestamp is the time when the delay request is arrived. And send

## ACK type=4

Type = 4

Data = error

No data if acknowledge is ok. Else send an error code.

There is different error type that exists:

- Error CRC

Type = 5

Data = id receiver

Ping type=5

Send in the data the device id you want to ping.

# CRC

CRC is calculed, it’s the sum of all the data+id+type.